Sun River Watershed Group

2016 Volunteer Monitoring Project

Sampling and Analysis Plan

July 7, 2016

Prepared by:

Kevin Stone, Coordinator

Sun River Watershed Group

PO Box 568

Choteau, MT 59404

Approved by:

______________________________________

Katie Steele, Project Manager (Montana DEQ)

______________________________________

Terri Mavencamp, QA Officer (Montana DEQ)

Table of Contents

1.0 INTRODUCTION	3

1.1 Project Area Overview	3

1.2 Project Goals and Objectives	3

1.3 Project Budget	4

2.0 Sampling Process	4

2.1 Study Design	4

Sampling Locations	5

Sampling Map	6

Sampling Timing	6

2.2 Sampling Methods	7

2.3 Field Forms	8

2.4 Laboratory Methods and Sample Handling Procedures	8

3.0 Quality Assurance/Quality Control	8

3.1 Quality Assurance and Quality Control Overview	8

3.2 Data Quality Indicators	10

Representativeness	10

Comparability	10

Completeness	10

Sensitivity	10

Precision, Bias and Accuracy for Water Samples	11

3.3 Training	12

3.4 Data Management, Record Keeping & Reporting	12

3.5 Project Team Responsibilities	13

3.6 Data Routing	13

4.0 ASSESSMENT RESULTS	13

4.1 Data Analysis	13

4.2 Data Communication	13

5.0	References	14

Appendix A - Project Budget	15

Appendix B – QA/QC Terms and Definitions	16

Appendix C – Quality Control Checklist	19

Laboratory QC	19

Appendix D – Data Qualifiers (Flags)	20

# 1.0 INTRODUCTION

## 1.1 Project Area Overview

The Sun River watershed is a sub-basin of the Missouri-Sun-Smith submajor basin, located in north-central Montana.  From its headwaters in the Rocky Mountains, the Sun River flows east for approximately 97.4 miles until it reaches the town of Great Falls, Montana, where it empties into the Missouri River.  Along the way, countless springs and numerous tributaries feed the Sun River.  The Sun River watershed, and the quality of the water it carries, has been significantly impacted by human activity, for better and for worse.   Sun River QAPP dated September 19, 2012 also contains useful data for this SAP.

The following are excerpts from the 2012 QAPP.  Several sections of the watershed were included on the state’s 2010 303(d) list.  The Sun River has an “impaired” designation, unable to support designated uses from Gibson Dam to Muddy Creek and also from Muddy Creek to the mouth at its confluence with the Missouri River.  Causes of impairment from Gibson Dam to Muddy Creek include alteration in stream-side or littoral vegetative covers, other flow regime alterations, sedimentation/siltation, and water temperature.  From Muddy Creek to the Missouri, causes include total nitrogen, total phosphorus, other flow regime alterations, sedimentation/siltation, and total suspended solids.  Other sections with impairment designations include Muddy Creek (headwaters to mouth), Ford Creek (from mouth to 2 miles upstream), Gibson Reservoir, Willow Creek Reservoir and Freezeout Lake.  More detailed information regarding 303(d) listings within the watershed can be found on Montana Department of Environmental Quality’s (DEQ’s) Clean Water Act Information Center website (http://cwaic.mt.gov/).

In December 2004, a document called the “Water Quality Restoration Plan and Total Maximum Daily Loads for the Sun River Planning Area” (WQRP) was finalized for the Sun River watershed.  The WQRP addressed these and other impairments on the Sun River, as well as impairments on several other waterbodies within the watershed.  The TMDL acknowledges the fact that conditions on all of the tributaries within the watershed “plays an important role in supporting beneficial uses” (pg 3 of the WQRP).

Previous monitoring summary by MSU 2009 report included:

- For the period of monitoring (2004-2009), overall salinity appears to have decreased slightly. Sun River at Augusta is relatively free of salinity, as reflected in conductivity measurements. Salinity increases downgradient, nearly tripling before the Sun River reaches Great Falls. It appears that each of the tributaries monitored is a measurable source of salinity. In a large majority of the cases of measurement, salinity (reflected in conductivity) is below the thresholds established in the TMDL. Exceptions occur when flows in tributaries are sourced primarily from seepage and ground water discharge, and are not augmented by either irrigation spillages or direct return flows.

- Total nitrogen appears to have decreased consistently during the period of record, although inspection of individual values identifies many occasions when the TN concentration exceeds the TMDL target. Efforts should be focused on identifying the sources/causes of elevated TN and initiating land resource management plans to give attention to reducing TN concentrations. TN appears to be heavily influenced by tributary inflows.

- Nitrate+nitrite-N clearly increases between Augusta and Great Falls. The period of record suggests a trend of increasing nitrate+nitrite-N concentration between 2004 and 2009. Muddy Creek, Mill Coulee, and Big Coulee all appear to be sources of significant contribution.  As with total nitrogen, efforts should be focused on identifying the sources/causes of elevated nitrogen and initiating land resource management plans to give attention to reducing nitrogen contributions.

- As with nitrogen, there appears to be a developing trend of increasing total phosphorus concentrations over time – in addition to a measurable and significant increase in total phosphorus between Sun River at Augusta and Sun River near Vaughn-Great Falls. The same recommendation applies with respect to total phosphorus as to nitrogen.

- Clearly total suspended sediment increases significant in Sun River between Augusta and Great Falls. There also appears to be a trend of increasing TSS during the period of record. While concentrations of TSS in Mill Coulee appear to have decreased during the 2004-2009 period, concentrations of TSS in Big Coulee appear to have increased during this same time period.

## 1.2 Project Goals and Objectives

The goal of this sampling project is to add to the existing Sun River water quality monitoring data set that will subsequently be used to assess trends in water quality and track progress towards reaching the goals of the Sun River TMDL plan and to determine the effects of the improvement projects.  The field data collected by SRWG will be considered along with all other readily available data obtained from federal, state and local agencies, other interested water quality organizations, and individuals in evaluating progress towards meeting water quality standards in the Sun River. A desirable outcome of continued monitoring is in identifying, evaluating and remediating the sources/contributing factors to nitrogen, phosphorus, and sediment loads in Muddy Creek, Mill Coulee, Big Coulee, and Adobe Creek. Data collected between 2004 and 2009 provides clear evidence of contributions to impairment from these tributaries. It is important to maintain monitoring stations to track changes in tributary impairment.

Continued monitoring provides data to evaluate whether projects have impacted sources/contributing factors to nitrogen, phosphorus, and sediment loads. This data and evaluation will assist in determining sources/contributing factors as well as the efficacy of various project approaches.

Table 1 – Project Goals, Research Questions and Objectives

| Goal | Question | Objective | Data analysis/Product |
| --- | --- | --- | --- |
| To evaluate whether water quality impairments noted in the Sun River TMDL are making improvement. | Have Nitrate+Nitrite, Total Nitrogen, Total Phosphorus, and Suspended Sediment Concentrations been reduced in the Sun River and selected tributaries? | Maintain data set of Nitrate+Nitrite levels over time. | Analyze trends over time and compare to TMDL goals. |
|  |  | Maintain data set of Total Nitrogen levels over time. | Analyze trends over time and compare to TMDL goals. |
|  |  | Maintain data set of Total Phosphorus levels over time. | Analyze trends over time and compare to TMDL goals. |
|  |  | Maintain data set of Suspended Sediment Concentrations over time. | Analyze trends over time and compare to TMDL goals. |
| To determine if large scale best management practices (BMP); irrigation water management (IWM) and livestock management; projects are working. | Have tributaries where water quality improvement projects have taken place show associated improvements in water quality? | Collect nutrient and TDS samples at 6 sites both during and following irrigation season. | Compare concentrations to existing data. Analyze trends over time and compare to TMDL goals. |
|  |  | Take photos of stream during each sampling event. | Visually estimate impact of BMP projects |

## 1.3 Project Budget

The total project budget is $ $1020.00. See Appendix A.

# 2.0 Sampling Process

## 2.1 Study Design

The project will sample on the Sun River from its upper reach near Augusta to the lower reach at Great Falls near the confluence with the Missouri River, as well as major tributaries that enter the Sun River.  Sampling from the selected sites enables us to collect water quality data upstream and downstream of agriculture and human activities. Samples will be collected prior to, during, and after peak flow events.  Monitoring at differing flow levels is important in the Sun River drainage because the farming and irrigation practices that contribute to some impairments vary throughout the season and at different flow levels.  Sampling under this study design is ongoing and will be continued with funding support.  Additional sampling in 2016 will be performed monthly starting in August and continuing through October.  Access to private land has been granted.

The Fairfield science teacher Rai Hahn, who has monitored water quality in the Sun River for more than 10 years, will sample all locations during the sampling events. The watershed coordinator will maintain contact with volunteers to schedule sampling dates and with the laboratory to acquire the appropriate bottles. After sampling, Mr. Hahn will quality check the samples by visually inspecting the sample bottles for physical integrity and ship them to the laboratory for analysis.

The sampling sites and parameters are appropriate because the TMDL indicated that agriculture contributions from the tributaries were the largest contributor to sediment and nutrients loads.  Best Management Practices efforts have reduced these contributions but monitoring is necessary to quantify improvement toward meeting water quality standards and to determine whether projects are meeting desired outcomes.

### Sampling Locations

Sites were identified and sampled in previous sampling efforts.  Previously used sites were chosen to allow for comparability between new and previously obtained sampling results.  Comparability was desired in order to facilitate trend analysis.

Table 2 - Sampling Locations*

| Site | Site Description | Latitude | Longitude | Analytes | Rationale for Site Selection |
| --- | --- | --- | --- | --- | --- |
| SUN-SUNR56 | Sun River near Augusta | 47.547861 | 112.366250 | Total Suspended Solids (TSS), Nitrate plus Nitrite as N, Total Phosphorus and Total Kjeldahl Nitrogen (TKN). | Near headwaters |
| SUN-SUNR50 | Sun River at Great Falls | 47.492028 | -111.334361 | same | At confluence |
| SUN-DUCKC01 | Big Coulee near Simms | 47.516972 | -111.887306 | same | Confluence with Sun |
| SUN-ADBEC01 | Adobe Creek near Ft Shaw | 47.510583 | -111.800611 | same | Confluence with Sun |
| SUN-MILCU01 | Mill Coulee near Sun River | 47.540611 | -111.705806 | same | Confluence with Sun |
| SUN-MUDYC57 | Muddy Creek at Vaughn | 47.561056 | -111.538306 | same | Confluence with Sun |

*These are proposed sampling locations; locations may change due to unforeseen access or other sampling issues.

### Sampling Map

Figure 1 - Map of Sampling Locations

### Sampling Timing

The 2016 data collection effort has included monthly sampling starting in April.  Funding will be used to continue monthly sampling in the period of August-October.

Table 3 - Sample Collection Timeframe of Lab parameters

| Date | Analytes | Reason for Date Selection |
| --- | --- | --- |
| April | Nitrate-nitrite-N, Total N, Total P, and TSS | Prior to high flow and irrigation |
| May | Nitrate-nitrite-N, Total N, Total P, and TSS | During high flows and prior to irrigation |
| June | Nitrate-nitrite-N, Total N, Total P, and TSS | During high flows and start of irrigation |
| July | Nitrate-nitrite-N, Total N, Total P, and TSS | During irrigation season |
| August | Nitrate-nitrite-N, Total N, Total P, and TSS | During irrigation season |
| September | Nitrate, Total N, Total P, Nitrate-nitrite-N, Total N, Total P, and TSS | During low flows and at end of irrigation |
| October | Nitrate-nitrite-N, Total N, Total P, and TSS | During low flows and after irrigation |

## 2.2 Sampling Methods

Table 4 - Sample Collection Methods

Preferred

Method

Alternative

Method

Preservation

Hold

Time

Justification

Field Parameters:

pH

YSI 556 multi-meter

Oakton Tester

NA

NA

Collected when samples are collected.

Temperature

YSI 556 multi-meter

Oakton Tester

NA

NA

Collected when samples are collected.

Specific Conductance (SC)

YSI 556 multi-meter

Oakton Tester

NA

NA

Cheap and easy surrogate for salinity.

Discharge (Q)

USGS Gage Data

Field Observation of

Gage w/ rating curve

NA

NA

Necessary to calculate loads; affects sediment,

salinity and all WQ parameters.

Turbidity

Hach

-

NA

NA

Erosion is a concern, meter already acquired,

hands-on opportunity for SRSC students.

Photos

Digital Camera

-

NA

NA

Tracking riparian condition; cheap and easy.

Lab Parameters:

Total Suspended Sediment  (TSS)

ASTM D3977-97

-

≤6 C

7 days

Erosion is a long term concern in watershed.

Nitrogen (total persulfate)

A4500-N C

A4500-N B

≤6 C

30 days

Muddy Creek exceeds standards.

Nitrate + Nitrite as N

EPA 353.2

A4500-NO3 F

H

2

SO

4

,  ≤6 C

28 days

Muddy Creek exceeds standards.

Phosphorus (total)

EPA 365.1

A4500-P F

H

2

SO

4

,  ≤6 C

28 days

Some tributaries exceed standards.

Sampling Methods

SRWG is responsible for water quality parameter sampling efforts, and will conduct sampling according to the SRWG SOP document, located in Appendix E.  A Site Visit Form (see Appendix E) will be completed for each site visit and will include all field data collected and an inventory of samples collected for analysis at the contracted laboratory.  Field parameters outlined in Appendix E and indicated on the Site Visit Form will be collected at each sampling event.  Site locations will be corroborated using the GPS coordinates, driving directions and photographs provided in the SOP document.  A GPS reading will be taken and recorded on the field visit form, using the NAD 1983 State Plane Montana datum, in decimal degrees to at least the fourth decimal. Photographs will be taken using a digital camera.  Field parameter data will be collected with a YSI 556, calibrated on the day of the sampling event, according to manufacturer instructions.  Site Visit Forms will be checked for completeness before leaving the sample site by Rai Hahn.

Flow (Discharge) Measurement

USGS uses automated gauges to collect flow data at Sun River at Augusta (SR-AG), Muddy Creek at Vaughn (MC-VHN), and Sun River at Vaughn (SR-GF).  USGS maintains and calibrates these gauges in accordance with their own procedures and standards.  DNRC creates rating curves for the gauges at the Big Coulee (BC-SM) and Mill Coulee (ML-200) sites via monthly visits May through October.  Fort Shaw Irrigation District also creates rating curves for the Adobe Creek (AC-200) site using this method.

Water Sample Collection and Handling Procedure

Grab samples will be collected for delivery to the DEQ-contracted lab for chemistry analysis using acid washed, polyethylene bottles provided by the testing laboratory. Table 5 details the analytical methods and handling procedures for each parameter. A detailed sampling schedule for each stream, is indicated in the Sampling Schedule and Parameters table of the SOP (Appendix B).

Bottles must be rinsed three times with stream water prior to sample collection in a well-mixed portion of each stream. During sampling, the sample bottle opening should face upstream and should be drawn through the water column once, carefully avoiding disturbance of bottom sediments.  Samples will be preserved in the field and stored on ice until shipment to the lab well in advance of the hold times listed above.

Quality control (QC) samples consisting of one blank and one duplicate will be collected each sample run and for each analyte.  A field blank is prepared by transporting laboratory-grade deionized (DI) water to the field (provided by the laboratory) and pouring it into sample containers provided by the lab. The blank will be prepared at the same time that the samples are collected from the stream.  A duplicate sample is a second, co-located stream sample collected at the same time in the same way that the regular stream sample is collected.  Duplicate and blank samples are labeled according to the labeling protocol below, which does not identify which sample is which to the lab. Blank and duplicate samples are preserved and handled and delivered to the lab in the same manner that regular samples are handled.

Sample labels should be filled out with Company (SRWG), the date, the time, and the sample ID.  The sample ID is very important and includes the year, the month, the day, the site ID and a letter indicating they type of sample (regular, duplicate, or blank).

Sample ID = [Year, Month, Day]_[Site ID]_[Sample-Type Letter]

A = Regular Sample

B = Duplicate Sample

C = Blank Sample

Sample ID Examples:

A regular sample collected at the Adobe Creek site on August 15th, 2016 would be labeled:

20160815_AC-200_A

A duplicate at the same place and time as above:

20160815_AC-200_B

A blank at the same place and time as above:

20160815_AC-200_C

Immediately following grab-sample collection, samples should be preserved with acids (as needed according to the tables in the Sampling and Laboratory Methods sections) and stored in a cooler on ice.  The DEQ-contracted analytical lab’s chain of custody (COC) forms will be used to document and track all samples collected during the project.  COCs will be completed for each set of samples submitted to the laboratory.  A sample COC can be found in the SOP document (Appendix B).

## 2.3 Field Forms

A Site Visit Form (see Appendix C) will be completed for each site visit and will include all field data collected and an inventory of samples collected for analysis at the contracted laboratory.

## 2.4 Laboratory Methods and Sample Handling Procedures

Table 4 – Monitoring Parameter Suite, Sample Handling, Analysis & Preservation

| Parameter | Preferred Method | Alternate Method | Required Reporting Limit ug/L | Holding Time Days | Bottle | Preservative |
| --- | --- | --- | --- | --- | --- | --- |
| Water Sample - Common Ions, Physical Parameters, Miscellaneous |  |  |  |  |  |  |
| Total Suspended Solids (TSS) | A2540 D | ASTM D3977-97 | 4000 | 7 | 500 ml HDPE | ≤6oC |
| Water Sample - Nutrients |  |  |  |  |  |  |
| Total Persulfate Nitrogen (TPN) | A4500-N C | A4500-N B | 40 | 7 | 250ml HDPE | ≤6oC |
| Total Phosphorus as P | EPA 365.1 | A4500-P F | 3 | 28 | 250ml HDPE | H2SO4 , ≤6oC or Freeze |
| Nitrate-Nitrite as N | EPA 353.2 | A4500-NO3 F | 10 |  |  |  |

# 3.0 Quality Assurance/Quality Control

Data needs to accurately represent the conditions in the watershed in order to be useful providing trend data for water quality within the watershed. Proper sample handling, processing, and assessment of data to ensure quality is required and should be examined thoroughly. Data quality objectives (DQOs) state the required quality of data for the intended use and data quality indicators (DQIs) are the specific criteria that data are assessed by to determine quality. These indicators are assessed by collecting quality control (QC) samples and then performing quality assurance (QA) checks on those samples. QC samples are the blank and duplicate samples collected in the field for evaluation of quality indicators. Once the results are processed for the QC samples, QA is the process of assessing the data through use of indicators to determine data quality.

## 3.1 Quality Assurance and Quality Control Overview

To inform water quality studies, data needs to accurately represent conditions in the watershed. Most projects require some degree of proper sample handling, processing, and data quality assessment, particularly when scientific or resource management questions are being investigated.

Quality Assurance (QA) is the overall management of a sampling program. It ensures the monitoring process, from the methods used to how data will be managed and analyzed, is adequate for the project to meet its objectives with a stated level of confidence. QA activities include developing a sampling and analysis plan, making sure that volunteers or staff is properly trained, and following standard operating procedures.

Quality control (QC) includes technical actions taken to detect and control errors. QC consists of developing measures and protocols to ensure sample collection and analyses are consistent and correct. If there is a problem, good QC will help to identify the problem. It also helps determine whether volunteer work is being performed correctly. QC activities may include collecting replicate samples for chemical analyses and the use of field blanks.

Data quality objectives (DQOs) are qualitative and quantitative statements that clarify the purpose of the study, define the most appropriate type of information to collect, determine the most appropriate conditions from which to collect that information, and specify tolerable levels of potential decision errors. Essentially, DQOs prompt monitoring project managers to determine what level of data quality is necessary to achieve the objectives of the project.

Data quality indicators (DQIs) are attributes of samples that allow for assessment of data quality. Because there are large sources of variability in streams and rivers, DQIs are used to evaluate the sources of variability and error and thereby increasing confidence in our data.

A list of Data Quality Assurance and Quality Control terms and definitions is included in Appendix B.

Provisions are in place to ensure sensitivity of data collected to differences in stream water quality and comparability of data collected to other datasets. These provisions include the collection of grab samples and field QC for submission to a certified laboratory and assessment of QC data relative to data quality indicators.

In order to ensure the highest degree of data completeness possible, volunteers need to fill out datasheets completely and review them before leaving a site.  Rai Hahn of Sun River Science Club will review datasheets for completeness and will follow-up with his student volunteers if any fields are illegible, inaccurate, or incomplete.

The study design has taken into account sample collection number and timing to ensure quality of data collected throughout the study site and the comparability of data collected to other sample events. These provisions include the collection of field QC samples and laboratory QC methods in accordance with EPA sampling methods. Data that does not meet quality criteria will be qualified appropriately in reporting and during the MT EQuIS submission process.

Lab quality objectives and QA/QC are described in further detail in the appendices.

## 3.2 Data Quality Indicators

This section describes for each data quality indicator (representativeness, comparability, completeness, sensitivity, precision and accuracy) how the sampling and analysis plan and study design aims to achieve data quality. Data quality indicator criteria are specified, where appropriate.

### Representativeness

Representativeness refers to the extent to which measurements represent an environmental condition in time and space. This project follows a judgmental sampling design in which spatial and temporal considerations were used to help ensure representativeness.

Spatial representation

The project’s sampling design helps achieve spatial representativeness whereas sampling sites were chosen to capture variability in land use, flow or other watershed characteristics that may be influencing water quality; monitoring site locations were selected based on use in previous sampling studies; sampling sites include key tributaries; and monitoring sites were selected along the entire length of the stream from headwaters to mouth.

Temporal representation

The project sampling design helps achieve temporal representativeness by collecting samples on a monthly basis and with temporal consistency.

### Comparability

Comparability is the degree to which different methods, data sets, and/or decisions agree or are similar. Comparability allows data users to determine the applicability of data to certain projects or decisions. For example, Montana DEQ may incorporate water chemistry data collected by volunteers if the methods, analytes and reporting limits are comparable to those that DEQ uses.

Comparability expresses the confidence with which one data set can be compared to another. To achieve a comparable result, both the field collection method and the analytical method must be comparable. This is achieved through the use of Standard Operating Procedures (SOPs – DEQ or USGS) for field collection and the use of the same analytical methods published by the EPA, APHA - Standard Methods, or USGS in the laboratory.  This sampling project utilizes sampling methods, analysis methods, and sample locations from previous years and studies in order to encourage comparability.

### Completeness

Completeness is a measure, expressed as a percentage, of the amount of data planned for collection compared to the amount actually collected. Prior to leaving a sampling site the Stream Team volunteers will be required to fill out a data sheet, which will be reviewed and signed by the field leader on site; this will reduce the occurrence of empty data fields. The overall project goal is 90% completeness. Because of the limited funding for laboratory analysis, collection of additional samples in the event of breakage of sample bottles en route to the laboratory is not planned.

Any loss of data due to site access issues, spillage, QC failures, or laboratory mistakes may result in no decisions being made due to insufficient data and a possible return trip to remote sites, or lessen the decision-making certainty. The project’s sampling design helps achieve completeness though the following provisions: all field forms will be reviewed for completeness prior to departure from the site; any sampling events that must be cancelled for any reason will be rescheduled; lab reports will be reviewed upon receipt to ensure that results for each sample submitted are received).

### Sensitivity

Sensitivity refers to the limit of a measurement to reliably detect a characteristic of a sample. Related to detection limits, sensitivity refers to the capability of a method or instrument to discriminate between measurement responses representing different levels of a variable of interest. The more sensitive a method is, the better able it is to detect lower concentrations of a variable. For analytical methods, sensitivity is expressed as the method detection limit (MDL).

Laboratory Sensitivity: Laboratories determine their method detection limits (MDLs) annually, and routinely check each method’s ability to achieve this level of sensitivity using negative controls (e.g., method blanks, continuing calibration Blanks, and laboratory reagent blanks). Sensitivity quality controls for all laboratory methods will follow the frequency and criteria specified in the analytical method or as described in the analytical laboratory’s Laboratory Quality Assurance Plan (LQAP).

Corrective Action: If the analytical method controls fail the specified limit, check with the laboratory to see how they addressed the non-conformance and qualify data as necessary.

### Precision, Bias and Accuracy for Water Samples

Bias is the degree of systematic error present in the assessment or analysis process. When bias is present, the sampling result value will differ from the accepted, or true, value of the parameter being assessed. Bias can occur either at sample collection or during measurement. Accuracy is the extent of agreement between an observed value (sampling result) and the accepted, or true, value of the parameter being measured. High accuracy can be defined as a combination of high precision and low bias. Precision measures the level of agreement or variability among a set of repeated measurements, obtained under similar conditions.

Evaluation of precision and accuracy for the water sampling portion of this project will consist of collecting and evaluating the results of field duplicates and field blank samples.

Precision: Field Duplicates

Field duplicates will be collected during this project and used to determine field and laboratory precision. Field duplicates consist of two sets of sample containers filled with the same water from the same sampling site. Duplicates will be collected two times per sampling season at different sites each time.  All duplicate samples will be collected at the same location. Field duplicate samples will be collected, handled and stored in the same way as the routine samples for laboratory shipment. Duplicates are used to determine field and laboratory precision.

Field duplicates will be used to evaluate data precision by calculating their relative percent difference (RPD):

RPD as % = ((D1 – D2)/((D1 + D2)/2)) x 100

where:

D1 is first replicate result

D2 is second replicate result

Precision for field QC samples will be assessed by ensuring that relative percent difference (RPD) between duplicates is less than 25%. If the RPD of field duplicates is greater than 25%, all data results from the duplicate pair’s parent sample that are less than 5 times the concentration in the duplicate sample will be flagged with a “J”.

Precision: Laboratory Duplicates

Energy Laboratories uses EPA approved and validated methods. Energy Laboratory’s standard operating procedures all require a method validation process including precision and accuracy performance evaluations and method detection limit studies. Internal laboratory spikes and duplicates are all part of Energy Laboratories quality assurance program; laboratory QA/QC results generated from this program are provided with the analytical results. The criteria used is 20% RPD for duplicate results greater than five times the MDL.

Accuracy: Field Blanks

Field blanks consist of laboratory-grade deionized (DI) water, transported to the field, and poured into a prepared sample container. Blanks are prepared in the field at the same time as the routine samples, and will be preserved, handled and analyzed in the same way as the routine samples. Blanks will be prepared twice per sampling session, at different sampling sites each time. Field blank samples are used to determine the integrity of the volunteer monitors’ handling of samples, the condition of the sample containers supplied by the laboratory, and the accuracy of the laboratory methods.

Accuracy for field QC samples will be assessed by ensuring that blank samples return values less than the lower reporting limit (shown in Section 3). If a blank sample returns a result greater than the threshold, all data for that parameter from that batch of samples will be qualified with a “B” flag. The exception is that data with a value greater than 10 times the detected value in the blank does not need to be qualified.

Accuracy: Laboratory

Accuracy of individual measurements will be assessed by reviewing the analytical method controls (i.e. Laboratory Control Sample, Continuing Calibration Verification, Laboratory Fortified Blank, Standard Reference Material) and the analytical batch controls (i.e. Matrix Spike and Matrix Spike Duplicate). The criteria used for this assessment will be the limits that Energy laboratory has developed through control charting of each method’s performance or based on individual method requirements.

Other

All samples will be checked to verify that they were processed within their specified holding times. Sample results whose holding time was exceeded prior to being processed will be qualified with an “H” flag.

Because of the limited funding for laboratory analysis, collection of additional samples in the event of data results that do not meet data quality objectives is not planned. If problems are linked to field crew sampling error, the data is either rejected or qualified, depending on the degree of the problem, and supplemental training will be provided prior to the next sampling event, as possible.

## 3.3 Training

All volunteers will be trained in all field methods, including field meters, sample collection and handling, prior to the initial sampling event. All volunteers have demonstrated adequate training as of 2016. Volunteers will demonstrate understanding of and proficiency in field methods to volunteer monitoring program manager(s) prior to sampling. Volunteers will be required to bring a copy of this SAP as well as any supplemental documentation of detailed field methods and/or standard operating procedures.

## 3.4 Data Management, Record Keeping & Reporting

The Project Manager is responsible for data management and record keeping, including the following activities that occur during or after the sampling is completed:

- Draft a brief synopsis of any SAP methodology derivations that occurred.

- Store and backup all data generated during this project, including field forms, laboratory reports obtained from the laboratories, electronic copied of field photographs, and written field notes.

- Review field forms for completeness and accuracy, especially Site Visit and Chain of Custody forms.

- Enter all laboratory data into MT e-WQX database.

- Maintain records of hours worked by volunteers for purposes of budget tracking.

Copies of laboratory analytical reports and Electronic Data Deliverable (EDD) spreadsheets will be provided by the DEQ contract analytical lab to both the Project Manager and to DEQ. All data will be entered by the Project Manager, or other specified party, into MT e-WQX database. Prior to entering data into the MT e-WQX database, the Project Manager will review the laboratory data in the following manner:

- Ensure lab results are within required reporting limits (including the laboratory QA/QC samples); if results are outside the reporting limits, the Project Manager will check with the laboratory to see how they addressed the non-conformance and qualify data as necessary.

- Complete the QC Checklist included in Appendix C.

- Assign appropriate data qualifiers provided in Appendix D to data, as needed, in both hardcopy and electronic form.

## 3.5 Project Team Responsibilities

Table 5 – Project Team Roles and Responsibilities

| Person | Role | Contact Information | Responsibilities | Training (optional) |
| --- | --- | --- | --- | --- |
| Rai Hahn | Volunteer | Removed from web version | Ensuring field forms are complete and accurate, Filling out COC form, Shipping samples. | Watercourse training |
| Kevin Stone | Project Leader | Removed from web version | Communicating with lab and DEQ, performing data QA and identifying data qualifiers, overall data management tasks per section 3.4, writing and submitting the final report to DEQ |  |

## 3.6 Data Routing

Data will be uploaded into the Montana Department of Environmental Quality (DEQ) Montana EqUIS database (http://deq.mt.gov/wqinfo/datamgmt/MTEWQX.mcpx) for eventual upload into EPA’s STORET database (http://www.epa.gov/storet/).

Spreadsheets with all field data collected by the volunteer will be emailed to Kevin Stone, who will review all laboratory and field data and conduct all QC procedures outlined in the Data Quality Control section of this document prior to data entry into the SRWG master spreadsheet.  The spreadsheet will ultimately be publically accessible via the SRWG website.  SRWG data will be housed in these spreadsheets and uploaded to EQuIS on an as-appropriate basis, with assistance from DEQ and/or Montana State University Extension Water Quality (MSUEWQ) personnel.

Table 6 – Data Routing Process

| Task | Information/Data | Primary Responsibility | Secondary Responsibility |
| --- | --- | --- | --- |
| Reviewing for completeness | field forms | volunteer | project manager |
| Spreadsheets | field forms | volunteer | project manager |
| upload and backup | digital site photos | project manager | n/a |
| lab coordination | sample chain of custody forms, electronic data deliverables | project manager | n/a |
| data entry into EQuIS | lab results, field measurements, site information | project manager | n/a |

# 4.0 ASSESSMENT RESULTS

## 4.1 Data Analysis

The possible results of the assessment are as follows:

1.  Sampling data reveals an increase in detected levels requiring SRWG to evaluate change of land use upstream or if SRWG needs to reevaluate BMP projects.

2.  Sampling data reveals a decrease in detected levels requiring SRWG to evaluate if this is a trend that needs the SRWG to accomplish more BMP projects.  BMP project tracking in water quality report will include where was the project located and what has been done differently, as well as how does WQ data demonstrate this change.

3.  Sampling data reveals the Sun River and tributaries are meeting water quality targets.  SRWG will request DEQ assistance to evaluate data and consider delisting Sun River from impaired stream list.

See section 1.2 for discussion of individual parameters.

## 4.2 Data Communication

Annual data summaries will be prepared for SRWG annual meetings by Kevin Stone.  In addition to reporting for the SRWG annual meeting, electronic copies of raw data and data summaries will be maintained on SRWG’s website.  In order to streamline this process, MSUEWQ has created an appendable Excel spreadsheet for each monitoring site that includes graphs of water quality parameters of interest using available historic data.   The addition of the current year’s water quality and discharge data, and some minor changes to the source data used to create the graphs is all that’s needed to bring these files up-to-date.

# References

# Appendix A - Project Budget

Projected Budget for Laboratory Analysis and Other Project Activities

| Description (Analyte or Activity) | Cost per Unit | Quantity | Total Cost |
| --- | --- | --- | --- |
| Nitrogen, Nitrate + Nitrite | $ 8 | 24 | $ 192 |
| Nitrogen, Total Persulfate | $15 | 24 | $360 |
| Phosphorus, Total | $10 | 24 | $240 |
| Solids, Suspended Sediment SSC @ 105 C | $ 8 | 24 | $ 192 |
| Shipping costs | $12 | 3 | $36 |

Total:  $1,020

# Appendix B – QA/QC Terms and Definitions

Accuracy. A data quality indicator, accuracy is the extent of agreement between an observed value (sampling result) and the accepted, or true, value of the parameter being measured. High accuracy can be defined as a combination of high precision and low bias.

Analyte. Within a medium, such as water, an analyte is a property or substance to be measured. Examples of analytes would include pH, dissolved oxygen, bacteria, and heavy metals.

Bias. Often used as a data quality indicator, bias is the degree of systematic error present in the assessment or analysis process. When bias is present, the sampling result value will differ from the accepted, or true, value of the parameter being assessed.

Blind sample. A type of sample used for quality control purposes, a blind sample is a sample submitted to an analyst without their knowledge of its identity or composition. Blind samples are used to test the analyst’s or laboratory’s expertise in performing the sample analysis.

Comparability. A data quality indicator, comparability is the degree to which different methods, data sets, and/or decisions agree or are similar.

Completeness. A data quality indicator that is generally expressed as a percentage, completeness is the amount of valid data obtained compared to the amount of data planned.

Data users. The group(s) that will be applying the data results for some purpose. Data users can include the monitors themselves as well as government agencies, schools, universities, businesses, watershed organizations, and community groups.

Data quality indicators (DQIs). DQIs are attributes of samples that allow for assessment of data quality. These include precision, accuracy, bias, sensitivity, comparability, representativeness and completeness.

Data quality objectives (DQOs). Data quality objectives are quantitative and qualitative statements describing the degree of the data’s acceptability or utility to the data user(s). They include data quality indicators (DQIs) such as accuracy, precision, representativeness, comparability, and completeness. DQOs specify the quality of the data needed in order to meet the monitoring project's goals. The planning process for ensuring environmental data are of the type, quality, and quantity needed for decision making is called the DQO process. Madison Stream Team Sampling and Analysis Plan Page 23

Detection limit. Applied to both methods and equipment, detection limits are the lowest concentration of a target analyte that a given method or piece of equipment can reliably ascertain and report as greater than zero.

Duplicate sample. Used for quality control purposes, duplicate samples are an additional sample taken at the same time from, and representative of, the same site that are carried through all assessment and analytical procedures in an identical manner. Duplicate samples are used to measure natural variability as well as the precision of a method, monitor, and/or analyst. More than two duplicate samples are referred to as replicate samples.

Environmental sample. An environmental sample is a specimen of any material collected from an environmental source, such as water or macroinvertebrates collected from a stream, lake, or estuary.

Field blank. Used for quality control purposes, a field blank is a “clean” sample (e.g., distilled water) that is otherwise treated the same as other samples taken from the field. Field blanks are submitted to the analyst along with all other samples and are used to detect any contaminants that may be introduced during sample collection, storage, analysis, and transport.

Instrument detection limit. The instrument detection limit is the lowest concentration of a given substance or analyte that can be reliably detected by analytical equipment or instruments (see detection limit).

Matrix. A matrix is a specific type of medium, such as surface water or sediment, in which the analyte of interest may be contained.

Measurement Range. The measurement range is the extent of reliable readings of an instrument or measuring device, as specified by the manufacturer.

Method detection limit (MDL). The MDL is the lowest concentration of a given substance or analyte that can be reliably detected by an analytical procedure (see detection limit).

Precision. A data quality indicator, precision measures the level of agreement or variability among a set of repeated measurements, obtained under similar conditions. Relative percent difference (RPD) is an example of a way to calculate precision by looking at the difference between results for two duplicate samples.

Protocols. Protocols are detailed, written, standardized procedures for field and/or laboratory operations.

Quality assurance (QA). QA is the process of ensuring quality in data collection including: developing a plan, using established procedures, documenting field activities, implementing planned activities, assessing and improving the data collection process and assessing data quality by evaluating field and lab quality control (QC) samples.

Quality assurance project plan (QAPP). A QAPP is a formal written document describing the detailed quality control procedures that will be used to achieve a specific project’s data quality requirements. This is an overarching document that might cover a number of smaller projects a group is working on. A QAPP may have a number of sample analysis plans (SAPs) that operate underneath it.

Quality control (QC). QC samples are the blank, duplicate and spike samples that are collected in the field and/or created in the lab for analysis to ensure the integrity of samples and the quality of the data produced by the lab.

Relative percent difference (RPD). RPD is an alternative to standard deviation, expressed as a percentage and used to determine precision when only two measurement values are available. Calculated with the following formula: RPD as % = ((D1 – D2)/((D1 + D2)/2)) x 100 Where: D1 is first replicate result D2 is second replicate result

Replicate samples. See duplicate samples.

Representativeness. A data quality indicator, representativeness is the degree to which data accurately and precisely portray the actual or true environmental condition measured.

Sampling and Analysis Plan (SAP). A SAP is a document outlining objectives, data collection schedule, methods and data quality assurance measures for a project.

Sensitivity. Related to detection limits, sensitivity refers to the capability of a method or instrument to discriminate between measurement responses representing different levels of a variable of interest. The more sensitive a method is, the better able it is to detect lower concentrations of a variable.

Spiked samples. Used for quality control purposes, a spiked sample is a sample to which a known concentration of the target analyte has been added. When analyzed, the difference between an environmental sample and the analyte’s concentration in a spiked sample should be equivalent to the amount added to the spiked sample.

Standard operating procedures (SOPs). An SOP is a written document detailing the prescribed and established methods used for performing project operations, analyses, or actions.

# Appendix C – Quality Control Checklist

Laboratory QC

___ Condition of samples upon receipt

___ Cooler/sample temperature within required range

___ Proper collection containers

___ All containers intact

___ Sufficient sample volume for analysis

___ Sample pH of acidified samples <2

___ All field documentation complete. If incomplete areas cannot be completed, document the issue.

___ Holding times met

___ Field duplicates collected at the proper frequency (specified in SAP)

___ Field blanks collected at the proper frequency (specified in SAP)

___ All sample IDs match those provided in the SAP. Field duplicates are clearly noted as such in lab results.

___ Analyses carried out as described in the SAP (e.g., analytical methods, photo documentation, field protocols)

___ Reporting detection limits met the project-required detection limit

___ All blanks were less than the project-required detection limit.

___ If any blanks exceeded the project-required detection limit, associated data is flagged.

___ Laboratory blanks/duplicates/matrix spikes/lab control samples were all within the required control limits defined within the SAP

___ Project DQOs and DQIs were met (as described in SAP)

___ Summary of results of OC analysis, issues encountered, and how issues were resolved addressed (corrective action)

___ Completed QC checklist before upload into DEQ’s EQuIS (or other) database.

# Appendix D – Data Qualifiers (Flags)

| Result Qualifier | Result Qualifier Description |
| --- | --- |
| B | Detection in field and/or trip blank |
| D | Reporting limit (RL) increased due to sample matrix interference (sample dilution) |
| H | EPA Holding Time Exceeded |
| J | Estimated: The analyte was positively identified and the associated numerical value is the approximate concentration of the analyte in the sample. |
| R | Rejected: The sample results are unusable due to the quality of the data generated because certain criteria were not met. The analyte may or may not be present in the sample. |
| D | Not Detected: The analyte was analyzed for, but was not detected at a level greater than or equal to the level of the adjusted Contract Required Quantitation Limit (CRQL) for sample and method. |
| UJ | Not Detected/Estimated: The analyte was not detected at a level greater than or equal to the adjusted CRQL or the reported adjusted CRQL is approximate and may be inaccurate or imprecise. |

# Appendix E – SRWG DOCUMENTS

SRWG Gear Checklist

General

1. SAP/SOP

2. Volunteer Waivers

3. Landowner Consent Form

4. YSI multi parameter meter or handheld meters

5. Calibration solutions

6. Calibration logs

7. Solution discard bottle

8. pH solutions (7 and 10)

9. EC 1413 μS/cm Standard

10. Deionized water squirt bottle

11. Kim wipes

12. Tap water for YSI storage

13. Calibration Log for YSI

14. Clipboard

15. Site Visit Forms

16. Pencils and Extra lead

17. Fine tip permanent marker

18. Broad tip permanent marker

19. Calculator

20. Batteries (4 C for YSI, 2 AA for GPS)

21. Duct tape

22. Camera

23. First aid kit

24. Bear spray plus transport container

25. Garmin eTrex GPS Unit

26. Multi-tool or screwdriver

27. Life Jacket (pfd)

28. Backpack to carry gear

Collecting Samples for Lab Analysis

1. Cooler from lab

2. Chain of Custody form (COC)

3. One set of sample bottles for each site and for any blank and duplicate QC samples

4. Sample Preservative (sulfuric acid)

5. Laboratory grade deionized water for blank samples

6. Plastic gloves

7. Safety glasses

8. Chain of Custody Forms

9. Ice

10. Packing tape for labels

Field Activity Checklist

1. Calibrate YSI meter before going to the field

2. Deploy YSI meter

3. Begin filling out field visit form

4. Label sample containers

5. Collect water samples

6. Collect YSI meter measurements

7. Take staff gauge readings (where applicable)

8. Prepare samples for shipping

9. Fill out chain of custody

10. Check that all forms are complete

11. Check that all gear is accounted for